| N230SA – Power Settings |  |  |  |  |
| --- | --- | --- | --- | --- |
| Takeoff & Climb |  |  |  |  |
| Takeoff Roll | Full throttle | Rotate ~7° up | 55 KIAS | Smooth rotation |
| Initial Climb | Full throttle | 7–8° up | 74 KIAS (Vy) | Trim early |
| Cruise Climb | Full throttle | ~5° up | 75- 85 KIAS | Vx 62 KIAS for obstacles |
| Cruise |  |  |  |  |
| Phase | Power | Pitch | Airspeed | Notes |
| Level Cruise | 2350–2500 RPM | 0–2° up | 100–115 KTAS | Depends on altitude & weight |
| Descent & Landing |  |  |  |  |
| Phase | Power | Pitch | Airspeed | Notes |
| 500 fpm | 2100–2200 RPM | 2 ° down | 100 KIAS | Adjust pitch for rate |
| Downwind (abeam) | 1700 RPM | Level | 90 KIAS | Flaps – 1st notch |
| Base | 1700 RPM | Slight down | 80 KIAS | Flaps – 2nd notch |
| Final | 1700 RPM | Nose-down | 70 KIAS (full flaps) | Flaps – 3rd notch |
| Crossing Threshold / Short Final | Power idle or as needed | Transition / End runway | 65 KIAS | DON’T CHANGE FLAPS SETTINGS |
| IFR Approach Settings - Precision Approach (ILS / LPV) |  |  |  |  |
| Segment | Power | Pitch | Airspeed | Notes |
| Intercept / Level at FAF | ~2150 RPM | Level | 90 KIAS | Configure early |
| Glideslope Descent | 1700 RPM | 2–3° down | 90 KIAS | ILS profile |
| Inside 500 ft | 1700 RPM | Nose-down | 80 KIAS | Smooth corrections |
| DA → Landing | As required | Nose-down | 70 KIAS (full flaps) | Transition to VFR landing profile |
| Non-Precision Approach (VOR / LNAV) |  |  |  |  |
| Segment | Power | Pitch | Airspeed | Notes |
| FAF Level Segment | ~2150 RPM | Level | 90 KIAS | Maintain altitude |
| Step-Down Descent | 1700 RPM | 3–4° down | 90 KIAS | Slightly steeper than ILS |
| MDA Level Segment | ~2100 RPM | Level | 90 KIAS | Hold MDA until MAP |
| MAP → Landing | As required | Nose-down | 70–80 KIAS | Transition to VFR landing |
| Circling (if required) |  |  |  |  |
| Segment | Power | Pitch | Airspeed | Notes |
| Circling Maneuver | 2150 RPM | Level | 80–90 KIAS | Maintain visual references |