| ARCHER III – Power Settings |  |  |  |  |
| --- | --- | --- | --- | --- |
| Takeoff & Climb |  |  |  |  |
| Takeoff Roll | Full throttle | Rotate ~7° up | 60 KIAS | Smooth rotation |
| Initial Climb | Full throttle | 7–8° up | 76 KIAS (Vy) | Trim early |
| Cruise Climb | Full throttle | ~5° up | 87 KIAS | Vx 64 KIAS for obstacles |
| Cruise |  |  |  |  |
| Phase | Power | Pitch | Airspeed | Notes |
| Level Cruise | 2400–2500 RPM | 0–2° up | 105–115 KTAS | Depends on altitude & weight |
| Descent & Landing |  |  |  |  |
| Phase | Power | Pitch | Airspeed | Notes |
| 500 fpm | 2100–2200 RPM | 2–3° down | 100–110 KIAS | Adjust pitch for rate |
| Downwind (abeam) | 2100–2200 RPM | Level | 90 KIAS | Begin configuring |
| Base | 1700–1800 RPM | Slight down | 80 KIAS | Add flaps as needed |
| Final | 1500–1700 RPM | Nose-down | 70 KIAS (full flaps) | Adjust power for glidepath |
| IFR Approach Settings - Precision Approach (ILS / LPV) |  |  |  |  |
| Segment | Power | Pitch | Airspeed | Notes |
| Intercept / Level at FAF | ~2100 RPM | Level | 90 KIAS | Configure early |
| Glideslope Descent | 1700–1800 RPM | 2–3° down | 90 KIAS | Rock-solid Archer ILS profile |
| Inside 500 ft | 1500–1700 RPM | Nose-down | 80–85 KIAS | Smooth corrections |
| DA → Landing | As required | Nose-down | 70 KIAS (full flaps) | Transition to VFR landing profile |
| Non-Precision Approach (VOR / LNAV) |  |  |  |  |
| Segment | Power | Pitch | Airspeed | Notes |
| FAF Level Segment | ~2100 RPM | Level | 90 KIAS | Maintain altitude |
| Step-Down Descent | 1500–1600 RPM | 3–4° down | 90 KIAS | Slightly steeper than ILS |
| MDA Level Segment | ~2100 RPM | Level | 90 KIAS | Hold MDA until MAP |
| MAP → Landing | As required | Nose-down | 70–80 KIAS | Transition to VFR landing |
| Circling (if required) |  |  |  |  |
| Segment | Power | Pitch | Airspeed | Notes |
| Circling Maneuver | 2000–2200 RPM | Level | 80–90 KIAS | Maintain visual references |